# A Microcontroller-Based Range Extender Module Supporting Legacy Device Compatibility

Problem Statement

Modern devices support 5GHz connectivity to a large extent. However, legacy devices and older hardware lack the required compatibility for the same.

On the other hand, institutional Wi-Fi coverage often suffers from physical attenuation and signal dead zones. Commercial repeaters are often expensive and consume loads of power.

Solution and Project Objectives

The main goal is to build a compact, energy-efficient 2.4 GHz Wi-Fi repeater capable of bridging network coverage using a single ESP32 development board. The repeater must also measure received signal strength indicators (RSSI), throughput, latency, and packet loss before and after range extension. We propose to simultaneously operate the module as a Wi-Fi Station (STA) connected to the existing router and a Soft Access Point (AP) for nearby devices, an arrangement supported by Espressif’s documentation.

System Architecture and Working Principle

The principle of Network Address and Port Translation (NAPT or simply NAT) plays a major role in this project. When the device to be connected (say, a laptop) requests a webpage, the packet travels through a rewrite process:

- Laptop sends an outbound request to the ESP32 NAT Layer which translates it. The ESP32 is its default gateway. At the ESP32, NAT translation happens.

- The ESP32 broadcasts the modified packet via its STA interface to the Main Router.

- The internet returns data to the Main Router, which forwards it to the ESP32.

- The ESP32 receives the reply on its STA interface, checks its internal NAT table, rewrites the destination IP back to the laptop, and broadcasts it over the AP interface to the laptop.

The idea is that the main router believes that the ESP32 is making all the web requests.

Hardware Required

ESP32 microcontroller, 2.4 GHz 3dBi SMA Antenna (if greater accuracy is required), USB-C cable.

Software Stack

Firmware: ESP32 NAT Router Firmware, or ESP-IDF (LwIP framework)

Configuration Interface: HTML5 / Embedded Web Server

Flashing Utility: esptool.py / ESP Web Tools

Expected Outcomes

- Range extension achieved with a bandwidth throughput of about 10 Mbps, which should be sufficient for general web browsing.

- About 15 metres of range extension

- Continuous classification of connection strength to enable the user to take action if connection drops substantially. This is achieved by metrics: RSSI, channel connection status, number of clients, and packet/throughput measurements